- Approved by 
Order of the Federal Environmental, Industrial and Nuclear Supervision Service No. 764
dated December, 29, 2011

PROVISION 
ON THE RADIOACTIVE SUBSTANCE INVENTORY RECONCILIATION IN THE ORGANIZATION

(RB-072-11)

- I. General

1. The Provision on the radioactive substance inventory reconciliation in the organization (hereinafter - the Provision) is reckoned among safety guides, contains the recommendations of the Federal Environmental, Industrial and Nuclear Supervision Service on the inventory reconciliation for radioactive substances (hereinafter - the RS) in the organization, serves as guidelines and represents no law or regulations.

2. This Provision shall be applied to inventory reconciliation for any radioactive substances subject to the state accounting and control.

3. The RS inventory reconciliation in the organization is performed for the following purposes:

1) to determine the available amount of radioactive substances (for the inventory items) in the organization;

2) to determine compliance of the actual parameters (weight and/ or volume of the radioactive substances in the inventory items) with the RS accounting data;

3) to prevent any losses, unauthorized use and theft of radioactive substances;

4) to check the integrity of the RS inventory items;

5) to compile the information on the presence of any radioactive substances for submittal to the public authorities and federal executive bodies engaged in supervision over atomic energy use and state regulation of safety.

The RS inventory items in the course of accounting and control include:

1) for sealed radionuclide sources (hereinafter - the SRS) - individual sources;

2) for depleted uranium products - individual products (except for the products and materials intended for manufacturing of fuel elements and fuel assemblies);

3) for unsealed radionuclide sources (hereinafter - the URS) in the form of individual items - individual sources;

4) for the URSs except for individual items - a certain amount of radioactive preparation in the package, or a lot of radioactive substances with homogeneous radionuclide composition;

5) for spent nuclear fuel (hereinafter - the SNF) - the removable section (the transport reactor core), an irradiated fuel assembly, an irradiated fuel element.

4. It is recommended to carry out the inventory reconciliation procedure in accordance with the requirements of the laws and regulations of the Russian Federation, national standards and other regulatory technical documents effective in the organization involved in the RS handling (hereinafter - the organization).

5. Inventory reconciliations in the organization are classified into initial, scheduled and unscheduled ones.

6. The initial inventory reconciliation in the organization shall be performed once upon its registration in the State System for Accounting and Control of Radioactive Substances and Radioactive Wastes (hereinafter - the RW), and in cases when no inventory reconciliation within the framework of the RS&RW State Accounting and Control System has been performed before. It is intended to determine the initial parameters of the RS characteristics. All radioactive substances present in the organization (including the SRSs, radiation sources, the URSs) shall be measured and identified in the course of initial inventory reconciliation.

The reporting information within the framework of the RS&RW State Accounting and Control System shall not be submitted to its information center in the course of initial inventory reconciliation for:

1) radionuclide sources in the fire detectors;

2) radiopharmaceutical preparations, immunoassay kits;

3) radioisotope generators for medical purposes except for radon generators;

4) radionuclide-labelled compounds;

5) radioisotope preparations and solutions based on short-lived radionuclides with the half-life of up to 60 days including iodine-125;

6) reference sources within any instruments.

Radioactive substances accounted in the state system for accounting and control of nuclear materials shall not be subject to initial inventory reconciliation.

Directors of any organizations performing initial inventory reconciliation are recommended to appoint the persons in charge of its performance and to establish inventory commissions consisting of the specialists with the required qualification.

7. It is recommended to carry out scheduled RS inventory reconciliations (including the radionuclides in spent nuclear fuel) in the organization within the structural units engaged in any activities with RS, at least once per a year.

8. It is recommended to carry out unscheduled inventory reconciliations in the following cases:

1) change of the accountable person;

2) change of the legal corporate form of the organization;

3) liquidation of the organization (structural unit);

4) restructuring of the organization (structural unit) or complete termination of any works with radioactive substances;

5) in case of any emergency situation (unauthorized access to the RS locations or storage facilities);

6) upon detection of any RS theft;

7) after elimination of any emergency situation consequences (accidents and other emergency situations);

8) in any other cases according to the decision of the organization director or the control body of the RS&RW State Accounting and Control System at the federal level, upon the request of the safety regulation agency for atomic energy use.

- II. Arrangement of inventory reconciliation

9. In order to carry out the RS inventory reconciliation in the organization it is recommended to issue the administrative document (an order or direction) from its director or any person authorized by him/ her (hereinafter - the organization director). It is recommended to define the following in the above-mentioned document:

1) the structural unit (units) of the organization where inventory reconciliation is to be performed;

2) the personal composition of the inventory commissions, the persons in charge of the inventory reconciliation;

3) the time limits for preparation of the radioactive substances subject to inventory reconciliation and the equipment, materials and documents necessary for inventory reconciliation;

4) the arrangements for performance of the inventory reconciliation procedures;

5) the time limits for inventory reconciliation and submittal of the reporting documents;

6) the time period upon expiry of which any transportation of the RS without the permit of the inventory commission chairperson shall be prohibited within the inventory reconciliation period (including shipping and receipt of RS).

10. The central inventory commission (hereinafter - the CIC) and working inventory commissions (hereinafter - the WICs) may be established in the organization.

As a rule, the CIC activities cover the entire organization, and activities of the WICs - the structural units of the organization.

According to the decision of the organization director several WICs or a single inventory commission combining the functions of the central and working commissions may be established.

It is recommended to appoint the CIC composition in the organization once per a year and WICs - prior to each inventory reconciliation. It is recommended to define the composition of the commission in such a way so that to ensure correctness, completeness and objectiveness of the inventory reconciliation results.

11. At least three persons should be included into the inventory commissions. In case of necessity the size of the commission may be increased, and its composition may be changed by the order (direction) of the organization director.

Absence of at least one member of the commission in the course of inventory reconciliation shall constitute the grounds for declaring of the inventory reconciliation results invalid.

12. It is recommended to appoint the following persons to the CIC:

1) the chairperson of the commission - the official responsible for arrangement of the RS accounting and control in the organization;

2) the members of the commission - the chief financial accountant or his/ her deputy, a representative of the RS&RW accounting and control service.

The following persons may be also appointed to the CIC:

1) RS accounting and measuring specialists;

2) representatives of the security service, the technical control department;

3) other specialists.

13. It is recommended to assign the following functions to the CIC:

1) organizational supervision over arrangement and performance of the inventory reconciliations in the structural units of the organization;

2) control of the WIC compliance with the requirements of the regulatory and technical documents for preparation and performance of the inventory reconciliation;

3) methodological assistance to the WIC in the course of the inventory reconciliation;

4) control of correct issuance and timely submittal of the inventory reconciliation reporting documents to the CIC;

5) review and consolidation of the provided materials subsequent to the inventory reconciliation results;

6) submittal of the final documents on the inventory reconciliation results according to the established format to the director of the organization;

7) control of fulfillment of the CIC resolutions and recommendations by the WICs.

14. Persons having passed the training and knowledge assessment with regard to RS accounting and control in accordance with the training programs shall be appointed to the WICs. In this case it is recommended to appoint the head of the structural unit where the inventory reconciliation is to be performed or his/ her deputy as the chairperson of the commission, and the representatives of the control and methodology group or any other specialists in RS accounting and control - as the members of the commission.

The following persons may be also appointed to the WIC:

1) specialists in computer-based RS accounting;

2) RS accounting and measuring specialists;

3) a representative of the security service;

4) a representative of the technical control department;

5) other specialists.

15. It is recommended to assign the following functions to the WICs:

1) the RS inventory reconciliation within the established time limits in accordance with the requirements of the laws and regulations of the Russian Federation, the national standards and any other regulatory and technical documents on the RS accounting and control in the organization;

2) registration of the information on all deficiencies of the RS accounting and control in the structural unit of the organization between inventory reconciliations and its submittal to the CIC;

3) issuance and submittal of the reporting documents.

16. Verification of the actual RS availability in the organization shall be carried out in mandatory presence of the persons responsible for the RS (warehouse keepers, custodians).

17. The personnel engaged in preparation and performance of inventory reconciliation as well as the members of inventory commissions are recommended to pass training and knowledge assessment in the inventory reconciliation procedures according to the procedure established in the organization.

- Inventory reconciliation procedure

18. It is recommended to include the following into the inventory reconciliation procedure:

1) preparation for inventory reconciliation;

2) inventory reconciliation in the structural unit;

3) documenting of the inventory reconciliation results (compilation of the inventory forms);

4) analysis of the inventory reconciliation results.

19. It is recommended to perform the following actions during preparation for inventory reconciliation in the organization:

1) to define the arrangements for inventory reconciliation;

2) to prepare the accounting and reporting documents required for inventory reconciliation;

3) to prepare the documents for the RS reclassification into radioactive wastes, the RS handover documents, the RS consumption documents;

4) to prepare the RS inventory items for inventory reconciliation in the structural unit and to bring them into the state convenient for inspection of their availability, integrity and identification numbers;

5) to compile the list of available RS inventory items in the organization (structural units of the organization) as of the inventory reconciliation date; the recommended format of this list is given in Appendix 1 to this Provision;

6) to prepare the equipment and measuring devices for inventory reconciliation in the structural unit.

20. It is recommended to perform the following procedures in the course of the RS inventory reconciliation in the structural unit of the organization:

1) reconciliation of the seal identifiers and their check for integrity and operability;

2) checking for availability, type and identifiers of the inventory items, checking of integrity for one-piece inventory items;

3) reconciliation of the attributes (characteristics that can be checked through the use of the visual and (or) instrumental control devices) of the RS inventory items with the data provided in the list of available RS inventory items;

4) confirmatory measurements of the RS characteristics and presence of any ionizing radiation from the inventory items;

5) measurement of the radioactive substances with absent accounting data or the accounting data credibility not confirmed by the state of the access control devices (hereinafter - the ACDs);

6) checking of the RS for compliance with the data sheets;

7) filling of the box reflecting the inspection results in the list of available RS inventory items;

8) issuance of the RS Inventory Reconciliation Certificate.

- Preparation of the documents for inventory reconciliation

21. In preparation of the documents for the RS inventory reconciliation the WIC members are recommended to check their composition and content.

The documents containing the required information on the RS in the course of inventory reconciliation, the RS measuring equipment and methods, the equipment used for the RS handling usually include:

1) administrative documents (the order or direction of the organization director with regard to inventory reconciliation, plans for preparation and performance of inventory reconciliation developed by the inventory commission);

2) the documents defining the inventory reconciliation procedures in the structural unit of the organizations (standards, guidelines, manuals, methodologies);

3) the documents defining the measurement procedures (standards, guidelines, manuals, methodologies, methodological recommendations);

4) the RS accounting documents;

5) the documents for the RS reclassification to another category;

6) the RS handover documents;

7) the RS consumption documents;

8) the reporting documents on the previous RS inventory reconciliation;

9) the operation documents for the equipment (manuals, data sheets, log books, certificates, verification and calibration reports).

It is recommended to submit the above-mentioned documents to the members of inventory commissions in the course of the RS inventory reconciliation in the structural unit of the organization.

22. It is recommended to apply the inventory reconciliation procedures established in the organization for sealed radionuclide sources in order to perform inventory reconciliation for radionuclide sources in the fire detectors, radiopharmaceutical preparations, immunoassay kits, radioisotope generators for medical purposes (except for radon generators), radionuclide-labelled compounds, radioisotope preparations and solutions based on short-lived radionuclides with the half-life of up to 60 days (including iodine-125), the reference sources in any instruments.

- Preparation of radioactive substances for inventory reconciliation

23. Prior to commencement of the inventory commission work the radioactive substances in the structural unit of the organization should be brought into the state convenient for checking of presence of the inventory items, their integrity, reading of their identification numbers as well as inspection of the presence and state of the seals installed on the RS packages.

24. All radioactive substances located in any vessels without identifiers prior to the inventory reconciliation commencement should be converted into inventory items (placed into special-purpose packages with seals having individual identifiers), and the necessary data should be included into the accounting documents.

The factory (serial) number marked on the product shall serve as the identification number for any inventory items in the form of RS-containing products without any package.

25. It is recommended to group the RS inventory items of the same type located in the storage facilities or temporary storage areas within the structural unit of the organization and to place them at the dedicated sites, racks or in the safe-type cabinets (lock boxes). It is recommended to indicate the numbering in well-visible places in order to facilitate identification of each RS inventory item location in the structural unit of the organization. Location of each inventory item within the storage facility should be registered on the storage facility chart, in the accounting documents and the electronic database of the RS and RW accounting and control system of the organization.

26.  It is recommended to provide individual areas for the WIC works in the structural units of the organization handling a great amount of different radioactive substances in order to facilitate inventory reconciliation.

27. It is recommended to develop the chart for the temporary RS storage facilities indicating the identification numbers of the inventory items and their locations (the rack number, the place on the rack, the lock box number).

28. It is also recommended to implement the following arrangements in the course of RS preparation for inventory reconciliation:

1) to stop any movements of radioactive substances (including their shipping and receipt) without the permit of the inventory commission chairperson within the inventory reconciliation periods;

2) to check for inclusion of all necessary data into the accounting documents.

- Compilation of the list of available radioactive substance inventory

29. It is recommended to compile the list of available RS inventory items developed and signed by the accountable person in the structural unit of the organization as the source document for subsequent inventory reconciliation procedures; this list should be submitted to the WIC of the structural unit or to the CIC of the organization. It should include the accounting data on each inventory item in the structural unit of the organization specified in the accounting documents of the structural unit (the RS registers, charts) as of the inventory reconciliation commencement moment. It is recommended to confirm credibility of the list of available RS inventory items as of the inventory reconciliation moment by its comparison with the data of the operation and accounting documents and databases.

30. It is recommended to group the inventory items of the same type included into the list of available RS inventory items documentarily into the sealed and unsealed RS groups.

31. It is recommended to submit the list of available RS inventory items in the structural unit of the organization in the format convenient for comparison of the actual and accounting data to the inventory commission by the inventory reconciliation commencement moment. In this case it is recommended to include all radioactive substances registered in the structural unit of the organization into this list.

32. In addition to the list of available RS inventory items it is also recommended to submit the issue and receipt accounting logs for sealed and unsealed radioactive substances, instruments, equipment and installations with radionuclide sources reflecting any facts of RS handover to other organizations or structural units of the organization from the previous inventory reconciliation moment to the inventory commission by the inventory reconciliation commencement moment.

Inventory reconciliation shall be performed for all radioactive substances in the structural unit of the organization regardless of their ownership (received for temporary use or custody).

Any information on the radioactive substances, instruments, equipment and installations with radionuclide sources handed over to other organizations or structural units of the organization and de-registered in this particular structural unit is not recommended for inclusion into the list of available RS inventory items in this structural unit.

- Preparation of documents, equipment and measuring devices for the radioactive substance inventory reconciliation

33. The documents confirming quality of the performed measurements and measurement protocols should be checked in the course of inventory reconciliation preparation and performance in order to assess compliance with the requirements for the measurement quality control.

34. The set of means and methods specified in the measurement program for each radioactive substance shall be used for measurements in the course of inventory reconciliation.

35. In case of necessity it is recommended to perform check measurements of the RS characteristics and additional calibration of the measuring system devices.

36. It is recommended to perform the following in the course of the equipment preparation for the RS inventory reconciliation:

1) to check the equipment for availability, state and compliance with the requirements of the operation documentation;

2) to check for availability and date of the latest equipment inspection.

37. It is recommended to check the RS measuring devices for compliance with the prescribed requirements in the course of their preparation for inventory reconciliation.

- III. Inventory reconciliation.

Inspection of the access control devices

38. Objectives of the ACD inspection:

1) to check for compliance of the sealing device types with the accounting data;

2) to check the state of the sealing devices;

3) to determine presence (absence) of any signs of unauthorized intervention and unauthorized access to the radioactive substances;

4) to check continuous control of access to the available information about the radioactive substances (storage and confirmation).

39. It is recommended to check the state of the installed seals within the intervals between RS inventory reconciliations with registration of the inspection results in documents. For the sealed radionuclide sources of the "design" hazard categories 1 and 2 it is recommended to check all inventory items with installed seals. For the sealed radionuclide sources of the "design" hazard categories 3, 4 and 5 it is recommended to make random selection of n seals from the entirety N of the inspected seals and conform their proper state.

In is recommended to define the random selection scope based on the requirement for confirmation of the proper state for at least 95% of seals with the confidence coefficient of 0.95. It is recommended to calculate the following value for determination of the random selection scope: n = [N (1 - (1 - 0.95)1/(0.05N))]+, where N is the number of seals installed in the structural unit with radioactive substances. In this case [ ]+ means rounding to the nearest larger integer. It is recommended to select the numbers of the seals to be checked at random from the numbered list of the whole entirety N of the inspected seals.

40. It is recommended to perform the inspection by visual or instrumental comparison of the inventory item identification attributes with the data specified in the accounting documentation and determination of any signs of unauthorized access and unauthorized intervention. In case of necessity other ACD inspections may be performed.

41. It is recommended to present the ACD inspection results in the RS Inventory Reconciliation Certificate. The recommended content of the RS Inventory Reconciliation Certificate for the structural unit of the organization is given in Appendix 2 to this Provision.

- Check of the correct data inclusion into the list of available radioactive substance inventory

42. The purpose of this check is to identify any ambiguities, omissions and other errors in the records that can occur during filling of the issue and receipt logs. In order to eliminate any errors in the source information it is recommended to check the RS accounting data entered to the accounting and control system for radioactive substances within the latest interval of time between inventory reconciliations (to the full extent or on the selective basis) by comparison of the data from different sources (supplementary sheets, packing lists, data sheets, certificates). It is recommended to perform screening inspection in case of any movement of radioactive substances in the structural unit within the period between inventory reconciliations and selective inspection - in case of no movement of radioactive substances in the structural unit within the period between inventory reconciliations.

43. It is recommended to check the data included into the list of available RS inventory items for correctness by reading and comparison of the attributes of the RS inventory items specified in the above-mentioned list for each RS inventory item with the actual attributes of the RS inventory items specified in the list of available RS inventory items for the previous inventory reconciliation, as well as by comparison with the data on the RS inventory items provided in the RS operation documentation (data sheets, log books, labels, certificates).

44. It is recommended to present the inspection results with indication of any detected punch items and proposals for their elimination in the RS Inventory Reconciliation Certificate.

- Confirmatory measurements of radioactive substances

45. In the course of inventory reconciliation it is recommended to perform confirmatory measurements of radioactive substances in order to confirm absence of any RS deficiency or excess.

For one-piece inventory items it is recommended to check the following:

1) compliance of the inventory items types and identifiers with the information specified in the data sheets;

2) integrity of the inventory items;

3) presence of any ionizing radiation from the inventory items;

4) activity of radioactive substances.

For dismountable inventory items it is recommended to check the following:

1) compliance of the inventory items types and identifiers with the information specified in the data sheets;

2) compliance of the sealing device types and states;

3) presence of any ionizing radiation from the inventory items;

4) activity of radioactive substances.

46. It is recommended to confirm availability of:

1) radionuclide sources in the fire detectors;

2) radiopharmaceutical preparations, immunoassay kits;

3) radioisotope generators for medical purposes except for radon generators;

4) radionuclide-labelled compounds;

5) radioisotope preparations and solutions based on short-lived radionuclides with the half-life of up to 60 days including iodine-125;

6) reference sources within any instruments.

For any radionuclide sources with the specified design operating lifetime it is recommended to make sure that this lifetime has not expired.

For any radionuclide sources subject to regular calibration it is recommended to confirm compliance with the calibration schedule.

47. The URSs present in each structural unit of the organization are recommended to:

1) be identified by measurement of the URS quantity and composition;

2) be controlled by in-service technical record-keeping and control checks for availability of any URSs in the form of inventory items according to the attributes as well as by regular reconciliation with the data of accounting documents;

3) be checked in the course of inventory reconciliation.

48. It is recommended to determine the scope of confirmatory measurements depending on the scope and results of the ACD inspections and control of the state for the RS inventory items. In case no accounting measurements have been performed for these radioactive substances within the time interval between the previous and the current inventory reconciliations and in the course of inventory reconciliation, credibility of the previous accounting measurement results is ensured through application of the access control devices, and no significant deviations of data are detected in the course of confirmatory measurements it is recommended to specify the previously measured values in the reporting documents.

- Presentation of the inspection results for radioactive substances

49. The RS inspection results shall be presented in a separate column in the list of available RS inventory items in the structural unit of the organization.

50. It is recommended to specify the accounting measurement results with regard to the amount and composition of radioactive substances in the inventory items in the accounting documents and the list of available RS inventory items (if any such measurements were performed in the course of inventory reconciliation).

51. The list of available RS inventory items shall be signed by all WIC members developing it. The lists of available RS inventory items obtained and approved in the course of this inventory reconciliation shall be the source data for the next inventory reconciliation.

- IV. Presentation of the inventory reconciliation results

52. Upon completion of each RS inventory reconciliation the RS Inventory Reconciliation Certificate shall be issued in the structural unit, signed by all members of the inventory commission as well as by the accountable person subsequent to familiarization with its content and then approved by the organization director.

53. It is recommended to specify the following the RS Inventory Reconciliation Certificate for the structural unit of the organization:

1) the inspection results with regard to maintenance of the accounting documentation and issuance of the reporting documentation, availability and storage conditions for the RS inventory items as well as the state of seals;

2) the information on the accounting and confirmatory measurements of radioactive substances performed in the course of inventory reconciliation;

3) identified violations and deficiencies of the RS accounting and control and the arrangements for their elimination implemented in the course of inventory reconciliation.

The conclusions are recommended to include the basic results of inventory reconciliation, particularly the statement of the actual RS availability compliance or non-compliance with the data specified in the list of available RS inventory items.

It is recommended to specify all identified violations of the RS accounting and control, to provide brief analysis of their causes and also the arrangements for their elimination implemented in the course of inventory reconciliation.

It is recommended to attach the protocols of all RS measurements performed in the course of inventory reconciliation to the RS Inventory Reconciliation Certificate for the structural unit.

54. It is recommended to include the list of available RS inventory items and any other documents issued subsequent to the inventory reconciliation into the RS Inventory Reconciliation Certificate for the structural unit in the form of appendices.

55. The lists of the available RS inventory items in the structural unit of the organization as of the inventory reconciliation date, the measurement protocols and other documents shall be compiled subsequent to the RS inventory reconciliation results for the structural unit of the organization and submitted to the person responsible for the RS accounting and control arrangement in the organization or to the RS accounting and control service of the organization (if any).

56. The inventory reconciliation results for the following items shall be recorded in accordance with the organization documents:

1) radionuclide sources in the fire detectors;

2) radiopharmaceutical preparations, immunoassay kits;

3) radioisotope generators for medical purposes except for radon generators;

4) radionuclide-labelled compounds;

5) radioisotope preparations and solutions based on short-lived radionuclides with the half-life of up to 60 days including iodine-125;

6) reference sources within any instruments.

57. In case of any RS deficiency (excess) detected in the course of inventory reconciliation, as well as in case of failure to confirm the properties of any inventory item recorded in the accounting documents in the course of confirmatory procedures that is indicative of potential RS deficiency/ excess in the inventory item the WIC chairperson in the structural unit of the organization should immediately inform the organization administration and the CIC chairperson.

It is recommended to carry out special-purpose investigations in order to determine the causes of such discrepancies, to define and record the available RS amount in the structural unit. The composition of the commission for investigations, the procedure for its work and the measurement plan shall be approved by the organization director.

58. It is recommended to describe any RS deficiency (excess) detected by the WIC in detail in the explanatory note which is to be attached to the RS Inventory Reconciliation Certificate for the structural unit of the organization. It is recommended to specify the deficiency (excess) detection date, time and location as well as the initials of the person having detected the deficiency (excess) in the explanatory note. The explanatory note shall be signed by all members of the commission. The investigation procedure shall be defined in the organization documents.

- Appendix 1
 to the Provision on the Radioactive Substance Inventory Reconciliation in the Organization approved by Order of the Federal Environmental, Industrial and Nuclear Supervision Service No. 764
dated December, 29, 2011

(reference)

List of 
available radioactive substance inventory items 
in ____________________________________________

(name of the organization or structural unit)

as of __ _________ 20 __

This List has been compiled on the basis of Order (Direction) No. _____ dated __ ________ 20__ with regard to RS inventory reconciliation.

All radioactive substances delivered to the organization (structural unit) have been registered, and all removed radioactive substances have been de-registered by the inventory reconciliation commencement.

The inventory items are brought into the state convenient for inspection of their availability, integrity and identification numbers.

| No. | Radioactive substance | Basic radionuclides | Inventory item type | Inventory item number | Sealing device name and number | RS amount |  |  |  |  | RS inspection results |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | activity, Bq | volume, m3 | weight, kg | inventory item, pcs. | location of the inventory item |  |
| 1 | 2 | 3 | 4 | 5 | 6 |  | 6 | 7 | 8 | 9 | 10 |

| Persons in charge of RS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | (position) |  | (full name) |  | (signature) |
|  | (position) |  | (full name) |  | (signature) |

- Appendix 2 
to the Provision on the Radioactive Substance Inventory Reconciliation in the Organization approved by Order of the Federal Environmental, Industrial and Nuclear Supervision Service No. 764
dated December, 29, 2011

(recommended format)

APPROVED BY

__________________________________

(position)

__________________________________

(full name)

__________________________________

(signature of the organization director)

__ ____________ 20__

Recommended content of the radioactive substance inventory reconciliation certificate for the structural unit of the organization

The present Certificate is issued to certify that the inventory commission consisting of:

___________________________________________________________________________

___________________________________________________________________________

has carried out the RS inventory reconciliation in structural unit No. .....

1. Basic results of the RS inventory reconciliation

___________________________________________________________________________

___________________________________________________________________________

2. Inspection results with regard to the RS accounting documentation maintenance and reporting documentation issuance

___________________________________________________________________________

___________________________________________________________________________

3. Results of the ACD state inspection

___________________________________________________________________________

___________________________________________________________________________

4. Inspection results with regard to the RS availability and storage conditions

___________________________________________________________________________

___________________________________________________________________________

5. Identified violations and deficiencies of the RS accounting and control

___________________________________________________________________________

___________________________________________________________________________

6. Proposals for elimination of any detected violations and deficiencies in the RS accounting and control

___________________________________________________________________________

___________________________________________________________________________

Chairperson of the commission

___________________________________________________________________________

(full name, signature)

Members of the commission

___________________________________________________________________________

___________________________________________________________________________

(full names, signatures)